3.6. If unidirectional composites are loaded in the fiber direction, calculate the ratios of fiber stress to matrix stress and fiber stress to composite stress when Vf = 10%, 25%, 50%, and 75%. Assume  Ef = 400 GPa and Em = 3.2 GPa.

3.7. Estimate EL, ET, GLT, and νLT, of glass–epoxy, graphite–epoxy, Kevlar–epoxy, and boron–aluminum composites with Vf = 25%, 50%, and 75%. Constituent properties are

For the purpose of calculations, assume all fibers to be isotropic.

3.8. A rod consists of a binder and two types of fibers with the following properties:

- What maximum load can this rod carry without rupturing any of the constituents? (Assume that the cross-sectional area of the rod = 10 cm2.)

- What is the maximum load the rod can carry?

- What constituent will rupture last?

- Plot the load–elongation curves for the rod to failure in load- maintained and elongation-maintained tests.

import matplotlib.pyplot as plt

# Material properties from the table (Density, Weight %, Young's Modulus, Tensile Strength)

materials = {

'Binder': {

'density': 1.3,  # g/cm^3

'weight_percentage': 35,  # %

'E': 3.5,  # Young's modulus in GPa

'sigma_max': 0.06  # Maximum tensile strength in GPa

},

'Fiber A': {

'density': 2.5,  # g/cm^3

'weight_percentage': 45,  # %

'E': 70,  # Young's modulus in GPa

'sigma_max': 1.4  # Maximum tensile strength in GPa

},

'Fiber B': {

'density': 1.6,  # g/cm^3

'weight_percentage': 20,  # %

'E': 6,  # Young's modulus in GPa

'sigma_max': 0.45  # Maximum tensile strength in GPa

}

}

# Total cross-sectional area of the rod

total_area = 10  # cm^2

# Calculate volume fractions based on weight percentages and densities

volume_fractions = {}

total_inverse_density_weight_ratio = 0

# First, compute the sum of weight% / density for normalization

for material, props in materials.items():

total_inverse_density_weight_ratio += props['weight_percentage'] / props['density']

# Now calculate the volume fractions and area for each material

for material, props in materials.items():

volume_fraction = (props['weight_percentage'] / props['density']) / total_inverse_density_weight_ratio

volume_fractions[material] = volume_fraction

materials[material]['area'] = volume_fraction * total_area  # Cross-sectional area

# Function to calculate maximum load before failure

def calculate_max_load(materials):

max_loads = {}

for material, properties in materials.items():

max_stress = properties['sigma_max']  # Maximum stress the material can handle

area = properties['area']  # Cross-sectional area of the material

# Calculate the maximum load the material can carry (force = stress * area)

max_load = max_stress * area  # Load in GPa * cm² = GPa * cm²

max_loads[material] = max_load  # Store the max load for each material

return max_loads

# Get the maximum loads for each material

max_loads = calculate_max_load(materials)

# Find the limiting load (smallest maximum load)

limiting_material = min(max_loads, key=max_loads.get)

limiting_load = max_loads[limiting_material]

print("Maximum loads each material can carry before failure:")

for material, load in max_loads.items():

print(f"{material}: {load:.2f} GPa*cm²")

print(f"{material} Area: {materials[material]['area']:.2f} cm²")

print(f"\nThe limiting material is '{limiting_material}' which fails first.")

print(f"The maximum load the rod can carry is {limiting_load:.2f} GPa*cm².")

# Plotting load vs. elongation for each material (just for illustration)

def plot_load_elongation(materials, max_load):

loads = []

elongations = {material: [] for material in materials}

increments = 50  # Number of increments for the plot

for load in range(1, increments+1):

current_load = (load / increments) * max_load

loads.append(current_load)

for material, props in materials.items():

elongation = current_load / (props['E'] * props['area'])  # Simplified elongation formula

elongations[material].append(elongation)

for material, elong in elongations.items():

plt.plot(loads, elong, label=material)

plt.xlabel("Load (GPa*cm²)")

plt.ylabel("Elongation")

plt.legend()

plt.title("Load vs Elongation for Different Materials")

plt.show()

# Plot load-elongation curves

plot_load_elongation(materials, limiting_load)

3.9. Two unidirectional composites are fabricated with glass fibers (Vf = 50%) and matrices A and B, whose stress–strain curves are shown in Figure 3.49. The glass fibers are elastic up to failure and have an elastic modulus of 70 GPa and ultimate tensile strength of 2.8 GPa. If the composites are stressed parallel to the fibers, calculate

- the composite stress at 1% and 4% strains for each composite, and

- minimum and critical fiber volume fractions for each composite.

Figure 3.49. Stress-strain curves for matrices A and B (Exercise Problem 3.9).

# Material properties

E_f = 70  # GPa for the glass fiber

sigma_u_f = 2800  # MPa (2.8 GPa converted to MPa)

V_f = 0.5  # Fiber volume fraction (50%)

V_m = 1 - V_f  # Matrix volume fraction

# Matrix stress from the graph at 1% strain and 4% strain for A and B

stress_matrix_A_1 = 17.5  # MPa

stress_matrix_A_4 = 35  # MPa

stress_matrix_B_1 = 35  # MPa

stress_matrix_B_4 = 70  # MPa

# Fiber stress at 1% and 4% strain using Hooke's Law (E_f * strain)

strain_1 = 0.01

strain_4 = 0.04

fiber_stress_1 = E_f * strain_1 * 1000  # Convert to MPa

fiber_stress_4 = E_f * strain_4 * 1000  # Convert to MPa

# Composite stress calculations using rule of mixtures

composite_A_1 = V_f * fiber_stress_1 + V_m * stress_matrix_A_1

composite_A_4 = V_f * fiber_stress_4 + V_m * stress_matrix_A_4

composite_B_1 = V_f * fiber_stress_1 + V_m * stress_matrix_B_1

composite_B_4 = V_f * fiber_stress_4 + V_m * stress_matrix_B_4

# Print the results

print(f"Composite A stress at 1% strain: {composite_A_1:.2f} MPa")

print(f"Composite A stress at 4% strain: {composite_A_4:.2f} MPa")

print(f"Composite B stress at 1% strain: {composite_B_1:.2f} MPa")

print(f"Composite B stress at 4% strain: {composite_B_4:.2f} MPa")

3.11. A tension link is made of an aluminum alloy. If aluminum is replaced by a unidirectional graphite–epoxy composite, calculate the required fiber volume fraction in the composite so that its longitudinal modulus matches the modulus of aluminum alloy. What is the percentage weight saving in this material replacement? Use the properties given in Table 1.1. The elastic modulus of epoxy is 3.5 GPa, and its density is 1.2 g/cm3.

# Given Data

E_aluminum = 70  # GPa

rho_aluminum = 2.7  # g/cm³

E_f = 390  # GPa (Graphite Fiber)

rho_f = 1.9  # g/cm³

E_m = 3.5  # GPa (Epoxy Matrix)

rho_m = 1.2  # g/cm³

# Step 1: Calculate the required fiber volume fraction (V_f)

E_c = E_aluminum  # We want the composite modulus to match aluminum

# Using Rule of Mixtures: E_c = V_f * E_f + (1 - V_f) * E_m

# Solve for V_f

V_f = (E_c - E_m) / (E_f - E_m)

# Matrix Volume Fraction (V_m)

V_m = 1 - V_f

# Step 2: Calculate the composite density

rho_c = V_f * rho_f + V_m * rho_m

# Step 3: Calculate the percentage weight saving

weight_saving = (rho_aluminum - rho_c) / rho_aluminum * 100

# Display Results

print(f"Required Fiber Volume Fraction (V_f): {V_f:.4f}")

print(f"Composite Density (rho_c): {rho_c:.4f} g/cm³")

print(f"Percentage Weight Saving: {weight_saving:.2f}%")